! VLAN Configuration Lab - SW1
! Create VLANs

enable
configure terminal

vlan 10

exit

vlan 20

exit

vlan 30
name VLAN30
exit

! Configure Access Ports

interface Ethernet0/1
switchport mode access
switchport access vlan 10
no shutdown
exit

interface Ethernet0/2
switchport mode access
switchport access vlan 20
no shutdown
exit

interface Ethernet0/3
switchport mode access
switchport access vlan 30
no shutdown
exit

! Verification Commands
 show vlan brief
show interfaces status
show running-config

end
write memory